TRIMP Direct Examination

This form is used to assess steel pipelines.

| Section 1 |
| --- |
| DE Location ID |  |  |  |  |  |  |  | 0042 C System E02 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HCA# |  |  |  |  |  |  |  |  |  | HCA0042_11192004_047A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Examination Number |  |  |  |  |  |  |  |  |  |  |  |  | E02 "B" Priority |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Work Request No. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 602968 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Division |  |  | CAD |  |  |  |  |  |  |  |  |  |  |  |  | District Number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0042 |  |  |  |  |  |  |  |  |  |  |  |  | Town or County |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Avondale |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | State |  |  |  |  | AZ |  |
| Tile Number |  |  |  |  |  | x366y886 |  |  |  |  |  |  |  |  |  |  |  |  | Address and/or Location |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | East of 4th Ave. on N side of Western Ave. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Inspection Company |  |  |  |  |  |  |  |  |  |  | Mears Engineering |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date GPS Synchronized |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |
| Field Location (from Top of Pipe) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Start: GPS X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -112.353970185 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Y |  |  |  | 33.435384021 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | End: GPS X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -112.353940964 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Y |  |  |  | 33.435388142 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GPS File Name |  |  |  |  |  |  |  |  | C System E02 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ECDA Region |  |  |  |  |  |  |  |  |  |  |  |  | A6 |
| Planned Examination Length |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10' |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Actual Examination Length |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Section 2 |
| Foreign Pipe in Excavation No Yes; |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Size |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Material |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Foreign Current Yes No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bond Present Yes No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | If Current Flow, To: SWG Foreign From: SWG Foreign |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CP Present Yes No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Anode Present Yes No % consumed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Environmental Conditions: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Temp |  |  |  |  |  |  |  |  |  |  |  |  | 104° F |  |  |  |  |  |  |  |  | Time 24-hr |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1239 |  |  |  |  |  |  |  |  |  |  |  |  | Weather Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Hot/Sunny |  |  |  |  |  |  |  |  |  |  |  |  |
| Soil Conditions: Dry Moist |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Bedding/Shading Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Native |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Rockshield Used Yes No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Soil Type: Clay Loam Sand Gravel Caliche Rocky Solid Rock |
| Flowable Backfill (Slurry) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pipe Data (as found in EMRS): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Nominal Size |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10" |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | InDiam |  |  |  |  |  |  |  |  |  | 10.136" |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Wthick |  |  |  |  |  |  |  |  | 0.307" |  |  |  |  |  |  |  |
| Grade |  | A |  |  |  |  |  |  |  |  |  | Yield |  |  |  |  |  |  |  |  | 30,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Coating |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | WkReqNo |  |  |  |  |  |  |  |  |  | 0102389541 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Installation Month |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Installation Year |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1948 |  |  |  |  |  |  |  |  | OpsSysName |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CSYS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Weld Seam: ERW Seamless Spiral Unknown Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Coating Types: Bare Pipe Grease Wax Tape Asphalt/Tar Wrap Pritec Fusion Bonded Epoxy |
| Plated Tape Wrap Field Applied Shrink Sleeve Extruded Poly Coating Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Coating Condition: Poor Fair Good Excellent |
| Type of Coating Damage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | % Damage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | O’clock/Position |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Holiday Detection Volt Setting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 12,000 V |  |  |  |  |  |  |  |  |  |  |
| Non-Corrosive Disbondment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Blistering Due to Corrosion |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | pH of Fluid in Blisters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| I have reviewed the procedures performed and have found them: Adequate *Inadequate |
| *If Inadequate, send comments and copy of WMS-WR to Engineering and Project Support Staff, LVA-581 |
| Inspected By |  |  |  |  |  |  | Rory C. Gutchak |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Inspection Date |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5/16/07 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Print and sign |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reviewed By |  |  |  |  |  |  | Joel Anderson |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date Reviewed |  |  |  |  |  |  |  |  |  |  |  |  |  | 10/31/07 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Print and sign |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Section 3 |
| Soil pH at Pipe Depth |  |  |  |  |  |  |  |  |  |  |  |  |  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (using Antimony half cell) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Soil Resistivity at Pipe Depth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2,000 |  |  |  |  |  |  |  |  |  |  |  | cm |  |  |
| Soil Chemistry Performed Yes No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Method used - Collins probe 4 pin method |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Results: |  |  |  | Chlorides |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ppm |  |  |  |  |  |  |  |  | Nitrates |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ppm |  |  |  |  |  |  |  |  |  |  |  |  | Sulfates |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ppm |  |  |  |  |  |  |  |
| Pipe to Soil from Start of Excavation: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 12 O’clock |  |  |  |  |  |  |  |  |  |  |  |  | -1.010 V |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 O’clock |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -1.010 V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6 O’clock |  |  |  |  |  |  |  |  |  |  |  |  | -1.010 V |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9 O’clock |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -1.010 V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bacterial Samples Taken Yes No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date of Incubation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bacterial Results: Scan and embed at the end of this document in Section 8. |
| Section 4 |
| Anomaly: Coating Defect, Pipe Damage, Corrosion and Pitting Measurements and Location (from Grid Sketch) None Found |
| Internal Corrosion (done by UT Inspection) External Corrosion was: General Pitting |
| Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cause of Corrosion: Galvanic Microbiological Stray Current SCC Improper CP Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| All external corrosion anomalies shall be identified and quantified below. The beginning of each anomaly will be referenced to a dig start. The upstream end of the dig is considered the “zero point” for measurements. The “anomaly number” identified below shall be referenced back to the anomaly sketch in Section 6 of this inspection report. |
| No. | Type of defect (Indicate Coating & Pipe Defects Separately) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Distance from Zero Reference (feet) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | O’clock Position |  |  |  |  |  |  |  |  |  |  | Longitudinal Length (inch) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Width (Circumferential Length) (inch) |  |  |  |  |  |  |  |  |  |  |  |  |  | Maximum Depth (inch) |  |  |  |  |  |  |  |  |  |  |  | Remaining Wall Thickness (inch) |  |  |  |  |  |
| Section 5 |
| Ultrasonic Thickness Testing. Distance from Zero Point; 12 o’clock (inch); 3 o’clock (inch); 6 o’clock (inch); 9 o’clock (inch). 1'; 0.312; 0.310; 0.302; 0.314. |

| Section 7 Section 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ANOMALY SKETCH Sketch the exposed section of piping with all anomalies superimposed on the grid below. Identify the direction you are looking down the pipe with a N/S/E/W indicator. The 6 o’clock position is the bottom dead center of the pipe. Identify individual anomalies and areas of joined anomalies per a standard numbering system starting from the upstream reference GPS stationing. Each individual anomaly or anomalous area identified below shall be quantified with spatial positioning, distance to reference, anomaly type, circumferential positioning, length, width, maximum depth and quantity in Section 4. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 10 9 8 7 6 5 4 3 2 1 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 0 1 2 3 4 5 6 7 8 9 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Indicate the direction of sight on the grid sketch. Indicate units of measure: feet inches |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Inspector’s Comments: Liquid found under the coating had a pH of 10. The depth of cover was 3’ 9” to the top of the pipe. The pipe was visually inspected and no defects were found. The pipe was primed and recoated with Polyken Tape. ICDA Scan: Min 0.305” / Max 0.313” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Section 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Take a minimum of two photos and attach or embed in this report.  Note location, date, and orientation of photo.

Sketch of dig site.  Note North on sketch.   Embed bacterial results (if taken).

10